- ABZORBIT PREMIUM ABSORBENT

- SECTION 1: PRODUCT AND COMPANY IDENTIFICATION

- PRODUCT NAME:         ABZORBIT PREMIUM ABSORBENT

SYNONYMS:                 Expanded Amorphous Alumina Silicate

- MANUFACTURER:         ABZORBIT, INC.

- ADDRESS:                     2628 N WEST BLVD / NEWTON NC 28658

- EMERGENCY AND CONTACT PHONE:   828-464-9944

- FAX PHONE:   828-464-4952

EMAIL: mail@abzorbit.com

- CHEMICAL NAME:         Expanded Amorphous Alumina Silicate

- CHEMICAL FAMILY:      Sodium Potassium

- CHEMICAL FORMULA: Aluminum Silicate of Variable Composition

- PRODUCT USE:             Sweeping Absorbent for liquid and semi-liquid spills.

- PREPARED BY:             ABZORBIT INC

- SECTION 2: COMPOSITION/INFORMATION ON INGREDIENTS

- INGREDIENT:  Expanded Amorphous Alumina Silicate

CAS NO.    93763-70-3       % WT:     100           % VOL:    100

OSHA PEL-TWA: 	15mg/m3  total particulate	5mg/m3    Respirable Fraction        ACGIH TLV-TWA:  5mg/m3

- SECTION 2 NOTES:

- May contain crystalline silica: Quartz 14808-60-7 Vol: <.1     ACGIH TLV-TWA:  .1MG/M3

- SECTION 3: HAZARDS IDENTIFICATION and FIRST AID MEASURES

- EMERGENCY OVERVIEW:  Dust contact with eyes may cause temporary irritation or redness.  The product has NOT been classified as a carcinogen by NTP or IARC.  Prolonged & repeated exposures to excessive concentrations of product dust in excess of the PEL/TLV can cause chronic pulmonary disease.

- ROUTES OF ENTRY: Inhalation, dust contact to eyes or mouth

- POTENTIAL HEALTH EFFECTS

- EYES:  Dust particulate can scratch and cause temporary irritation. Rinse thoroughly.

- Do not rub eyes.  If irritation continues consult a physician.

- SKIN: N/A

INGESTION: Inert mineral silicate but non hazardous.

INHALATION:  Remove to fresh air. Drink water to clear throat and blow nose to evacuate dust.

- Windy conditions a mask is recommended

ACUTE HEALTH HAZARDS:  Transitory upper respiratory or eye irritation

CHRONIC HEALTH HAZARDS:  Prolonged & repeated exposure to excessive concentrations of product dust in excess of the     PEL/TLV can cause chronic pulmonary disease

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE:  Pre-existing upper respiratory and lung disease such as, but not limited to bronchitis, emphysema and asthma.

- NOTES TO PHYSICIANS OR FIRST AID PROVIDERS:  Abzorbit is in inert mineral silicate.  Eye exposure treatments would follow those of any silicate or sand product.

- SECTION 4: FIRE-FIGHTING MEASURES

- FLAMMABLE LIMITS IN AIR, UPPER:  Non Flammable

- FLASH POINT: N/A

AUTO IGNITION TEMPERATURE: N/A

- NFPA HAZARD CLASSIFICATION: N/A

- EXTINGUISHING MEDIA: None if product unused. If used to absorb flammable liquids, then consult MSDS of liquid absorbed

- SECTION 5: ACCIDENTAL RELEASE MEASURES

- ACCIDENTAL RELEASE MEASURES: No special precautions. Vacuum or sweep. Avoid creating excessive dust.  Maintain good housekeeping procedures regarding nuisance dust.  If product has been used to absorb a liquid, consult the MSDS of that liquid for additional information.

- SECTION 6:  HANDLING AND STORAGE

- HANDLING AND STORAGE:  Handle to avoid creating excessive dust

OTHER PRECAUTIONS:    Maintain good housekeeping practices.  Remove from floor as soon as liquid is absorbed. Dispose of according to the MSDS of the liquid being absorbed

- SECTION 7:  EXPOSURE CONTROLS/PERSONAL PROTECTION

- EYE PROTECTION:  Goggles NIOSH approved recommended

- VENTILATION: Provide adequate exhaust ventilation Use sufficient natural or mechanical ventilation to keep dust level below PEL

RESPIRATORY PROTECTION: We recommend the use of NIOSH approved respirators for protection for specified TLV

SKIN PROTECTION: Normally not necessary (Consult MSDS of liquid being absorbed)

OTHER PROTECTIVE CLOTHING OR EQUIPMENT: Normally not necessary

WORK HYGIENIC PRACTICES: Good Housekeeping practices

- SECTION 7 NOTES: If dusty conditions exist, NIOSH approved protective equipment is advisable

- SECTION 8:  PHYSICAL AND CHEMICAL PROPERTIES

- APPEARANCE: Dry White Aggregate

ODOR: No order

PH (Other):              6.5-7.5

- BOILING POINT: N/A

- MELTING POINT:    F:  2000

- FREEZING POINT: N/A

- VAPOR PRESSURE (mmHg):     N/A

- VAPOR DENSITY (AIR = 1):   N/A

- SPECIFIC GRAVITY (H2O = 1): .08-.2

- EVAPORATION RATE: N/A

- SOLUBILITY IN WATER: Insoluble in water and most acids

- SECTION 9: STABILITY AND REACTIVITY

- STABILITY: Stable

- CONDITIONS TO AVOID (STABILITY): N/A

- INCOMPATIBILITY (MATERIAL TO AVOID): Hydrofluoric Acid forms a toxic silicon tetra fluoride gas

- HAZARDOUS POLYMERIZATION:       Will not occur

- SECTION 10:  TOXICOLOGICAL INFORMATION

- TOXICOLOGICAL INFORMATION: Prolonged and repeated exposure to excessive concentrations of this product’s dust or any nuisance dust can cause chronic pulmonary disease. Dust contact with eyes may cause temporary scratchiness or redness.  This product has not been classified as a carcinogen by NTP or IARC.

- SECTION 11:  DISPOSAL CONSIDERATIONS

- WASTE DISPOSAL METHOD: As manufactured is acceptable for landfill.  If contaminated by liquid or semi liquid the manner of disposal would be governed by that liquid or semi liquid. Consult the MSDS of the contaminated product.

RCRA HAZARD CLASS: Not defined as Hazardous waste per 40CFR261

- SECTION 11 NOTES:  Contact your local landfill operator to determine if you can dispose of the liquid you have absorbed if that liquid is deemed hazardous. (IE:  gasoline, oil, antifreeze, paint, etc) Consult the MSDS of the liquid being absorbed.

- SECTION 12 REGULATORY INFORMATION

- U.S. FEDERAL REGULATIONS

- TSCA (TOXIC SUBSTANCE CONTROL ACT): Material listed in TCSA inventory, Excluded from reporting requirements

CERCLA (COMPREHENSIVE RESPONSE COMPENSATION, AND LIABILITY ACT): Material is no reported under CERCLA

SARA TITLE III (SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT):

311/312 HAZARD CATEGORIES: Hazard Categories- non found

313 REPORTABLE INGREDIENTS: none

- STATE REGULATIONS: N/A

- INTERNATIONAL REGULATIONS: N/A

- DISCLAIMER: The information on this Data Sheet represents our current data and best opinion as to the proper use in handling of this material under normal conditions.  Any use of this material which is not in conformance with the Data Sheet or which involves using the material in combination with any other process is the responsibility of the user.  However, no warranty or representation with the respect to such information is intended or given.